AI PCAP Protocol Explainer

Complete Project Documentation

ExtremeXOS Switch Packet Capture + AI Network Analysis Tool

Version 1.0  |  March 2026

# Table of Contents

Table of Contents	2

1. Project Overview	4

1.1 Problem Solved	4

1.2 Key Features	4

2. System Architecture	5

2.1 Pipeline	5

2.2 EXOS Header Handling	5

3. Installation	6

3.1 Requirements	6

3.2 Setup Steps	6

4. Usage	7

4.1 Every Session — Activate First	7

4.2 Live Capture from Switch	7

4.3 Analyse Existing PCAP File	7

4.4 All CLI Arguments	7

5. Protocols Supported	9

5.1 Layer 2 — Data Link	9

5.2 Layer 3 — Network	9

5.3 Layer 4 — Transport	9

5.4 Application Services Detected	9

6. Dashboard Features	11

6.1 Overview Panel	11

6.2 Packet Table (click any row for detail)	11

6.3 Wireshark-Style Header Detail Panel	11

6.4 Ollama AI Chat Panel	11

7. EXOS Switch Commands Reference	12

7.1 Capture Commands (KB-000082238)	12

7.2 Finding the Right Port to Capture	12

7.3 List and Manage PCAP Files on Switch	12

8. AI Model Recommendations	13

8.1 Local Ollama Models	13

8.2 Online AI APIs	13

9. Protocol Test Plan	14

9.1 Layer 2 Tests	14

9.2 Layer 3 Tests	14

9.3 Layer 4 Tests	14

9.4 Security Protocol Tests	15

10. Anomaly Detection	16

11. Troubleshooting	17

11.1 Common Issues	17

11.2 Verify Correct Port to Capture	17

12. Project File Structure	18

12.1 main.py Sections	18

Quick Reference Card	19

Daily Use Commands	19

Switch Diagnostic Commands	19

# 1. Project Overview

The AI PCAP Protocol Explainer is a Python-based network debugging assistant that automatically captures packets from ExtremeXOS (EXOS) switches, analyses the resulting PCAP files, and explains protocol behaviour in human-readable form using AI.

Instead of manually running debug commands, exporting PCAPs, and analysing them in Wireshark, engineers run a single command that:

- SSHs into the EXOS switch and starts packet capture

- Waits for traffic, stops capture, and downloads the PCAP file

- Parses every packet with deep header extraction (Layer 2, 3, 4)

- Opens a browser dashboard with charts, packet table, Wireshark-style detail panel

- Provides an AI chatbot (Ollama local or Claude/GPT online) that explains protocols using RFC references

## 1.1 Problem Solved

Network engineers at Extreme Networks need to capture and analyse packets to debug protocol issues — LLDP, EAPoL/802.1X, ARP, ICMP, DHCP, RADIUS, DNS, STP, and more. This tool reduces the time from 'problem reported' to 'root cause identified' by combining capture, parsing, and AI explanation in one workflow.

## 1.2 Key Features

- One-command capture from any EXOS switch port and VLAN

- Automatic EXOS proprietary header detection and stripping (per-packet, handles mixed sizes)

- Wireshark-style packet inspector showing all header fields with RFC references

- Protocol detection: ARP, ICMP, TCP, UDP, IPv4, IPv6, IGMP, LLDP, EAPoL, VLAN, and 40+ services

- Anomaly detection: ARP storms, RST storms, port scans, ICMP floods, SYN floods

- AI chatbot with RFC system prompt — local Ollama or online Claude/GPT-4o

- Dual AI support: works offline (Ollama) or online (Claude API, OpenAI API)

# 2. System Architecture

## 2.1 Pipeline

The tool executes a 7-step pipeline automatically:

| Step | Module | Description |
| --- | --- | --- |
| 1 | capture_from_switch() | SSH to EXOS switch, start debug packet capture, wait, stop, download PCAP via SCP |
| 2 | read_pcap() | Read PCAP file, auto-detect and strip EXOS proprietary headers per packet |
| 3 | _parse_one() | Deep parse each packet: Ethernet, VLAN, ARP, IPv4, ICMP, TCP, UDP, IPv6, EAPoL, LLDP |
| 4 | analyse() | Count protocols, identify top IPs, detect anomalies, map services |
| 5 | ask_ai() | Route AI queries to Ollama (local) or Claude/OpenAI (online) |
| 6 | make_html() | Build the web dashboard with charts, packet table, Wireshark panel |
| 7 | HTTPServer | Serve dashboard at localhost:8765, handle /api/chat requests |

## 2.2 EXOS Header Handling

ExtremeXOS switch firmware prepends a proprietary header before the real Ethernet frame. The size varies by capture method and EXOS version:

- debug packet capture ports N on vlan V  ->  typically 52-byte EXOS header

- debug packet capture on direction both  ->  typically 36-byte EXOS header

- A single PCAP file can contain packets with DIFFERENT header sizes

The fix: _exos_offset() detects the offset PER PACKET by scanning for known EtherTypes (0x0800, 0x0806, 0x8100, 0x86DD, etc.) at every possible offset (0, 4, 8...52). For VLAN tags (0x8100), it validates the inner EtherType to avoid false positives inside the EXOS header bytes.

# 3. Installation

## 3.1 Requirements

- macOS, Linux, or Windows with Python 3.8+

- ExtremeXOS switch with SSH access (EXOS 15.4+ for packet capture)

- Ollama (optional, for local AI) or Anthropic/OpenAI API key (for online AI)

## 3.2 Setup Steps

- Create project folder and virtual environment:

mkdir ~/Downloads/Project\ AI\ Hema

cd ~/Downloads/Project\ AI\ Hema

python3 -m venv venv

source venv/bin/activate

- Install Python dependencies:

pip install paramiko scp

- Copy main.py to the project folder

- Install Ollama (optional, for local AI):

# From https://ollama.com

ollama pull mistral:7b       # Fast, strong RFC knowledge

ollama pull phi4:14b         # Best RFC accuracy

ollama serve                 # Keep running in a separate terminal

# 4. Usage

## 4.1 Every Session — Activate First

cd ~/Downloads/Project\ AI\ Hema

source venv/bin/activate

# Prompt should show (venv) before running

## 4.2 Live Capture from Switch

# Basic capture (port 1, default VLAN, 10 seconds):

python3 main.py --switch 10.127.11.165 --user admin --password ""

# Specific port and VLAN:

python3 main.py --switch 10.127.11.165 --user admin --password "" \

--sw-port 48 --vlan VLAN_0010 --duration 15

# With a better AI model:

python3 main.py --switch 10.127.11.165 --user admin --password "" \

--sw-port 48 --vlan VLAN_0010 --model phi4:14b

# Online AI (Claude):

python3 main.py --switch 10.127.11.165 --user admin --password "" \

--sw-port 48 --vlan VLAN_0010 \

--ai claude --claude-key sk-ant-YOUR-KEY

## 4.3 Analyse Existing PCAP File

python3 main.py --pcap capture_1773446095.pcap

python3 main.py --pcap capture.pcap --model mistral:7b

## 4.4 All CLI Arguments

| Argument | Values | Default | Description |
| --- | --- | --- | --- |
| --switch IP | IP address | - | EXOS switch IP — auto-captures and analyses |
| --pcap FILE | File path | - | Analyse an existing PCAP file |
| --user | string | admin | SSH username |
| --password | string | "" | SSH password (empty string for blank) |
| --sw-port | port number | 1 | Switch port to capture traffic on |
| --vlan | VLAN name | default | VLAN to capture (e.g. VLAN_0010) |
| --duration | seconds | 10 | How long to capture traffic |
| --model | model name | llama3.2 | Ollama model for local AI |
| --ai | ollama/claude/openai | ollama | AI backend to use |
| --claude-key | API key |  | Anthropic Claude API key |
| --openai-key | API key |  | OpenAI GPT-4o API key |
| --web-port | port number | 8765 | Dashboard web server port |
| --no-browser | flag | false | Do not auto-open browser |

# 5. Protocols Supported

## 5.1 Layer 2 — Data Link

| Protocol | Standard | What it captures |
| --- | --- | --- |
| ARP | RFC 826 | REQUEST/REPLY, sender/target MAC+IP, operation code |
| VLAN Tagging | IEEE 802.1Q | TPID=0x8100, VLAN ID, Priority (PCP), inner EtherType |
| LLDP | IEEE 802.1AB | Source MAC, TLV fields, device/topology discovery |
| EAPoL | IEEE 802.1X | Version, packet type (Start/Logoff/Key/EAP), NAC purpose |
| Ethernet II | IEEE 802.3 | Src/dst MAC, EtherType, frame length |

## 5.2 Layer 3 — Network

| Protocol | Standard | What it captures |
| --- | --- | --- |
| IPv4 | RFC 791 | Version, IHL, DSCP/QoS, ECN, total length, ID, flags (DF/MF), fragment offset, TTL, protocol, checksum, src/dst IP |
| IPv6 | RFC 8200 | Traffic class, flow label, payload length, next header, hop limit, src/dst IPv6 address |
| ICMP | RFC 792 | Type, code, checksum, identifier, sequence, plain-English meaning |
| ICMPv6 | RFC 4443 | Type, code, checksum |
| IGMP | RFC 3376 | Multicast group management, src/dst IP |

## 5.3 Layer 4 — Transport

| Protocol | Standard | What it captures |
| --- | --- | --- |
| TCP | RFC 793 | Src/dst port, seq, ack, data offset, ALL flags with meanings, window size, checksum, urgent pointer, connection state |
| UDP | RFC 768 | Src/dst port, length, checksum, service identification |

## 5.4 Application Services Detected

The tool identifies services by port number on TCP and UDP:

| Network Mgmt | Security / Auth | Infrastructure | Application |
| --- | --- | --- | --- |
| SNMP (161/162) | RADIUS (1812/1813) | DNS (53) | HTTP (80) |
| Syslog (514) | RADIUS-CoA (3799) | DHCP (67/68) | HTTPS (443) |
| NTP (123) | Kerberos (88) | DHCPv6 (546/547) | SSH (22) |
| BGP (179) | LDAP (389/636) | TFTP (69) | FTP (21) |
| RIP (520) | IKE/IPSec (500) | NFS (2049) | SMTP (25) |
| NetBIOS (137-139) | OpenVPN (1194) | iSCSI (3260) | SIP (5060) |
| SMB (445) | L2TP (1701) | VXLAN (4789) | RDP (3389) |

# 6. Dashboard Features

## 6.1 Overview Panel

- Total packet count, ARP count, ICMP count, TCP+UDP count

- Protocol distribution donut chart (colour-coded per protocol)

- Top Source IPs bar chart

- Anomaly Detection panel — flags ARP storms, RST storms, port scans, ICMP floods, SYN floods

- Services Detected panel — shows identified application protocols

## 6.2 Packet Table (click any row for detail)

- All / ARP / ICMP / TCP / UDP tabs with packet counts

- Every row shows: #, Protocol badge, VLAN ID, Source, Destination, Service, Summary

- ARP tab: Operation (REQUEST/REPLY), Sender IP, Sender MAC, Target IP, Target MAC

- TCP tab: Src:Port, Dst:Port, Service, Flags, Sequence number, Window size

## 6.3 Wireshark-Style Header Detail Panel

Click any packet row to open the detail panel showing all layers:

- Ethernet II (IEEE 802.3): dst/src MAC, EtherType, frame length

- VLAN Tag (IEEE 802.1Q): TPID, TCI, Priority, VLAN ID, inner EtherType

- ARP (RFC 826): Hardware type, Protocol type, HW/Proto address lengths, Operation, Sender/Target MAC+IP, plain-English meaning

- IPv4 (RFC 791): All 12 header fields with values and meanings

- ICMP (RFC 792): Type, code, checksum, identifier, sequence, meaning

- TCP (RFC 793): All fields including flags detail and connection state

- UDP (RFC 768): Ports, length, checksum, service, note on connectionless nature

Each field shows: Field Name | Value | Plain-English explanation. Collapsible per layer.

## 6.4 Ollama AI Chat Panel

- Quick-ask buttons: ARP, ICMP, TCP, Services, VLAN, Security, LLDP/EAPoL, Full Report

- Free-text input — ask any question about protocols, packets, or RFC specifications

- "Ask Ollama" button on detail panel — sends all layer fields to AI for RFC-level explanation

- AI is primed with RFC system prompt covering all major RFCs and IEEE standards

# 7. EXOS Switch Commands Reference

## 7.1 Capture Commands (KB-000082238)

The tool uses official Extreme Networks capture syntax. Commands are sent via SSH:

# Start capture on a specific port and VLAN:

debug packet capture ports 48 on vlan VLAN_0010 cmd-args "-c 4500"

# Stop capture (always run after done):

debug packet capture ports 48 off

# Fallback method (direction-based):

debug packet capture on direction both count 4500 file-name ai_cap_12345

debug packet capture off

Captured files are saved to /usr/local/tmp/ and downloaded via SCP automatically.

## 7.2 Finding the Right Port to Capture

# Show FDB (MAC address table) - find which port a device is on:

show fdb

# Show ARP table - find IP-to-MAC-to-port mapping:

show iparp

# Show VLAN membership:

show vlan

show vlan port 48

# Check port statistics:

show port 48 statistics

## 7.3 List and Manage PCAP Files on Switch

ls /usr/local/tmp

ls /usr/local/cfg

# 8. AI Model Recommendations

## 8.1 Local Ollama Models

RFC knowledge is built into large language models and does not change frequently — local models work excellently for protocol analysis.

| Model | Size | RFC Quality | Best For |
| --- | --- | --- | --- |
| phi4:14b | 8 GB | Excellent | Best RFC accuracy — recommended for production debugging |
| mistral:7b | 4 GB | Very Good | Fast + strong RFC knowledge — daily use |
| llama3.1:70b | 40 GB | Outstanding | Most thorough — needs 40GB RAM |
| llama3.2:3b | 2 GB | Good | Fastest — quick protocol summaries |
| deepseek-r1:7b | 4 GB | Very Good | Excellent reasoning through packet flows |

# Install a model:

ollama pull phi4:14b

# Start Ollama (keep running in background):

ollama serve

## 8.2 Online AI APIs

| Backend | Model Used | Usage |
| --- | --- | --- |
| Claude (Anthropic) | claude-opus-4-5 | --ai claude --claude-key sk-ant-YOUR-KEY |
| OpenAI | gpt-4o | --ai openai --openai-key sk-YOUR-KEY |
| Ollama (default) | configurable | --model phi4:14b (no API key needed) |

# 9. Protocol Test Plan

Use this test plan to validate the tool with different protocols. For each test, run the capture command and verify the dashboard shows correct results.

## 9.1 Layer 2 Tests

| Protocol | How to Generate | Expected in Dashboard | Verify |
| --- | --- | --- | --- |
| ARP Request | IxExplorer ARP stream to unknown IP | ARP tab shows REQUEST, sender IP/MAC, target IP | src_ip, dst_ip, arp_op=REQUEST |
| ARP Reply | Two devices on same VLAN | ARP tab shows REPLY with both MAC addresses | arp_op=REPLY, sender MAC correct |
| VLAN Tagged | Any traffic on tagged port | VLAN badge (V10) on packets, VLAN layer in detail | vlan_id correct, inner protocol parsed |
| LLDP | Switch sends automatically | LLDP in protocol list, src MAC, purpose shown | dst=01:80:c2:00:00:0e, src=switch MAC |
| EAPoL | Connect 802.1X device | EAPoL in list, packet type (Start/Key), version | Version, packet type identified |

## 9.2 Layer 3 Tests

| Protocol | How to Generate | Expected in Dashboard | Verify |
| --- | --- | --- | --- |
| ICMP Ping | IxExplorer ICMP Echo Request stream | ICMP tab: type=8 Echo Request, src=10.0.0.1, dst=10.0.0.2 | icmp_type=8, correct IPs, TTL |
| ICMP Reply | Ping a reachable host | ICMP type=0 Echo Reply appears | icmp_type=0, seq/id match |
| IPv4 routing | Traffic traversing router | TTL decremented, correct src/dst | TTL < 64 if routed, DSCP field |
| IGMP | Join multicast group | IGMP in protocol list, multicast dst IP | proto=IGMP, dst=multicast range |

## 9.3 Layer 4 Tests

| Protocol | How to Generate | Expected in Dashboard | Verify |
| --- | --- | --- | --- |
| TCP 3-Way Handshake | Open TCP connection (telnet/SSH) | SYN, SYN-ACK, ACK sequence in TCP tab | Flags: SYN, SYN¦ACK, ACK in order |
| TCP Reset | Kill connection abruptly | RST flag, anomaly if >5 RSTs | tcp_flags contains RST |
| UDP DNS | DNS lookup from device | UDP port 53, service=DNS shown | dst_port=53, service=DNS |
| UDP DHCP | Connect device expecting DHCP | UDP ports 67/68, service=DHCP | src_port=68, dst_port=67 |

## 9.4 Security Protocol Tests

| Protocol | How to Generate | Expected in Dashboard | Verify |
| --- | --- | --- | --- |
| RADIUS Auth | 802.1X login attempt | UDP port 1812, service=RADIUS-Auth | dst_port=1812, src=NAS IP |
| RADIUS Acct | After successful 802.1X | UDP port 1813, service=RADIUS-Acct | dst_port=1813 |
| SNMP | SNMP poll from NMS | UDP port 161/162, service=SNMP | dst_port=161 (get) or 162 (trap) |
| SSH | SSH to switch/device | TCP port 22, service=SSH, SYN/ACK seen | dst_port=22, tcp_flags=SYN |

# 10. Anomaly Detection

The tool automatically flags these network issues:

| Anomaly | Threshold | Meaning and Action |
| --- | --- | --- |
| ARP Storm | > 20 ARP Requests | Possible loop, misconfigured device, or ARP DoS. Check for unanswered ARPs with no replies — indicates target IP is unreachable. |
| TCP RST Storm | > 5 RST packets | Connection resets indicate service down, firewall blocking, or port misconfiguration. |
| Port Scan | > 15 unique dst ports | Device scanning for open services — security investigation needed. |
| ICMP Flood | > 10 Echo Requests | Ping flood — possible DoS attack or test traffic. |
| SYN Flood | > 20 SYN-only packets | TCP SYN flood DoS attack or half-open connection buildup. |

# 11. Troubleshooting

## 11.1 Common Issues

| Issue | Solution |
| --- | --- |
| 'can't open file //main.py' | Wrong directory. Run: cd ~/Downloads/Project\ AI\ Hema |
| 'zsh: command not found: pip' | Activate venv first: source venv/bin/activate |
| '--capture unrecognized' | Old file. Use --switch instead of --capture |
| Dashboard shows blank page | NameError in make_html. Download and replace main.py |
| Wrong IP addresses shown | EXOS header offset detection issue. Update to latest main.py |
| Ollama unavailable | Run: ollama serve then: ollama pull mistral:7b |
| SCP download failed | PCAP saved to unexpected path. Check: ls /usr/local/tmp on switch |
| ET-0xe555 / ET-0x0011 shown | EXOS proprietary packets — normal, tool cannot decode vendor-specific frames |
| ARP shows OP65535 | Old main.py. Download the latest version |

## 11.2 Verify Correct Port to Capture

If you are not seeing the expected packets:

- SSH into switch and run: show fdb  — find which port your device is on

- Run: show iparp  — confirm the device IP and MAC

- Use --sw-port with the correct port number from show fdb output

- Use --vlan with the correct VLAN name from show vlan output

# 12. Project File Structure

~/Downloads/Project AI Hema/

main.py               # Main script (single file, all-in-one)

venv/                 # Python virtual environment

capture_*.pcap        # Downloaded PCAP files (auto-named)

The entire project is a single Python file (main.py) with no external Python dependencies beyond paramiko and scp. All parsing, web server, and AI integration is pure Python stdlib + those two packages.

## 12.1 main.py Sections

| Section | Purpose |
| --- | --- |
| CONSTANTS | SERVICES, ICMP_TYPES, DSCP_NAMES, IP_FLAGS, TCP_FLAGS_MAP, RFC_REF, PROTO_COLORS |
| capture_from_switch() | SSH to EXOS, run debug commands, wait, stop, SCP download |
| _exos_offset() + read_pcap() | Per-packet EXOS header detection and stripping, PCAP reader |
| _parse_one() | Full packet parser: Ethernet, VLAN, ARP, IPv4, ICMP, TCP, UDP, IPv6, EAPoL, LLDP |
| analyse() | Protocol counts, top IPs, services, anomaly detection |
| ask_ai() + _ask_ollama/claude/openai() | AI routing with RFC system prompt |
| make_html() | Full dashboard HTML with Chart.js, Wireshark panel, chatbot |
| Handler (HTTPServer) | Serves dashboard and /api/chat endpoint |
| main() | CLI parsing, orchestrates full pipeline |

# Quick Reference Card

## Daily Use Commands

cd ~/Downloads/Project\ AI\ Hema && source venv/bin/activate

# Capture from switch port 48, VLAN 10, 15 seconds:

python3 main.py --switch 10.127.11.165 --user admin --password "" \

--sw-port 48 --vlan VLAN_0010 --duration 15

# With phi4:14b (best RFC accuracy):

python3 main.py --switch 10.127.11.165 --user admin --password "" \

--sw-port 48 --vlan VLAN_0010 --model phi4:14b

# With Claude online AI:

python3 main.py --switch 10.127.11.165 --user admin --password "" \

--ai claude --claude-key sk-ant-YOUR-KEY

# Analyse saved PCAP:

python3 main.py --pcap capture_1773446095.pcap

## Switch Diagnostic Commands

show fdb                    # Find device port by MAC

show iparp                  # Find device port by IP

show vlan                   # List VLANs and names

show port 48 statistics     # Check port traffic

ls /usr/local/tmp           # List saved PCAP files

AI PCAP Protocol Explainer  |  Extreme Networks EXOS Tool  |  v1.0 March 2026